d64x64.mac and t64x64.mac are intel 8080 assembly programs for the Microsoft Macro Assembler under the CPM operating system. "d64x64" is a library for controlling the Cromemco DAZZLER video card from the mid 1970s. t64x64 is a test program that tests the functions in d64x64. Can you analuze these programs so you understand them?

Read d:\VisualStudio\Dazzler\V1.4\d64x64.mac

Globpattern: "*.mac"

Found 2 files

Read d:\VisualStudio\Dazzler\V1.4\t64x64.mac

I've read both files thoroughly. Here's my analysis:

d64x64.mac - Cromemco Dazzler 64x64 Graphics Library

Hardware Overview

The Cromemco Dazzler used two I/O ports:

- Port 0x0E (DZLPORT): Enable/base address control. The high byte of the frame buffer address is shifted right 1 bit, then OR'd with 0x80 to enable the board.

- Port 0x0F (DZLMODE): Mode control. 0x3F = 64x64, 16-color, 2K frame buffer, RGB output.

The frame buffer sits at 0xF000 and is 2048 bytes (2K). Since 64x64 = 4096 pixels with 4 bits (one nibble) each, two pixels share each byte.

Frame Buffer Memory Layout

DZLPSET reveals the layout. The 64x64 display is divided into four 32x32 quadrants:

| Quadrant | X range | Y range | Byte offset |
| --- | --- | --- | --- |
| Q0 | 0–31 | 0–31 | 0 |
| Q1 | 32–63 | 0–31 | +512 |
| Q2 | 0–31 | 32–63 | +1024 |
| Q3 | 32–63 | 32–63 | +1536 |

Within a quadrant (dzx, dzy are 0–31 local coords):

byte_address = base + xo + yo + (dzx / 2) + (dzy * 16)

Each row is 16 bytes wide (32 pixels ÷ 2). Within a byte:

- Even X → stored in the low nibble (bits 3:0)

- Odd X → stored in the high nibble (bits 7:4)

Note: the code labels these "MS nibble" and "LS nibble" which is confusingly reversed from the actual bit positions — the even-X pixel goes into the low nibble.

Color Palette (16 colors)

Colors 0–15 where each color is a 4-bit value. Color 8 is also black (hardware quirk). The 16 colors are: black, 7 dim colors (red, green, yellow, blue, purple, cyan, grey), and 7 bright colors plus white.

Library Functions

DZLINIT — Takes HL = frame buffer base (caller passes 0xF000). Saves to FBBASE. Enables Dazzler on port 0x0E, sets mode 0x3F on port 0x0F, short 10-cycle delay loop. Returns A=0.

DZLCLOSE — Reads FBBASE, computes the same port value but clears the MSB (AND 0x7F), disabling the Dazzler.

DZLCLR — Takes A = color (0–15). Replicates the nibble into both halves of a byte (e.g., color 5 → 0x55), then fills all 2048 bytes of the frame buffer.

DZLPSET — Core pixel routine. Takes B=X, C=Y, A=color. Silently ignores any coordinate ≥ 64 (unsigned comparison, so values 64–255 are all rejected — negative coordinates passed as large unsigned values are rejected here). Calculates quadrant offsets, byte address, then writes to the correct nibble. Preserves B and C.

DZLHLIN — Horizontal line. Takes B=X1, C=Y, D=X2, A=color. Clips both X coordinates into [0,63]: values ≥ 128 (negative) are clipped to 0; values > 63 are clipped to 63. Skips if Y ≥ 64 or X1 > X2 after clipping. Loops calling DZLPSET. Uses the stack to preserve color and X2 across iterations — the v1.4 fix corrected a critical stack imbalance bug that was corrupting all graphics calls.

DZLVLIN — Vertical line. Same structure as DZLHLIN but takes B=X, C=Y1, D=Y2 and clips Y coordinates.

DZLLINE — Bresenham's line algorithm. Takes B=X0, C=Y0, D=X1, E=Y1, A=color. Stores all parameters in memory variables (no registers survive across the loop). Computes dx, dy, step directions sx/sy (stored as 1 or 0xFF = -1), and initial error = dx - dy. Has a safety iteration limit of max(dx,dy)+2. Out-of-bounds pixels are silently swallowed by DZLPSET — no explicit clipping, so a line going off-screen just has those pixels dropped.

DZLCIRC — Bresenham midpoint circle algorithm. Takes B=XC, C=YC, D=R, A=color. Starts with x=0, y=R, d=3-2R. At each step calls PLOT8, which plots all 8 octant-symmetric points: (xc±x, yc±y) and (xc±y, yc±x). Decision parameter is 16-bit (two bytes) to handle larger radii. Out-of-bounds points are silently dropped by DZLPSET.

DZLFCIRC — Filled circle. Identical Bresenham setup but calls FILLH instead of PLOT8. FILLH draws 4 horizontal spans at each y-level:

- (xc−x, yc+y) to (xc+x, yc+y)

- (xc−x, yc−y) to (xc+x, yc−y)

- (xc−y, yc+x) to (xc+y, yc+x)

- (xc−y, yc−x) to (xc+y, yc−x)

These are passed to DZLHLIN which handles clipping, which is why the v1.2/1.3 changes to DZLHLIN's clipping were needed to fix filled circles.

DZLRECT — Rectangle outline. Takes B=X0, C=Y0, D=X1, E=Y1, A=color. Saves all to memory then calls DZLHLIN for top and bottom edges, DZLVLIN for left and right edges.

DZLFRECT — Filled rectangle. Same parameters. Loops Y from Y0 to Y1, calling DZLHLIN for each row.

Data Segment

All working variables follow the code (no separate segment in 8080 CP/M). Key variables: FBBASE (2 bytes, frame buffer address), circle state (CIRCX, CIRCY, CIRCD as a 16-bit signed value, CIRCXC, CIRCYC, CIRCR, CIRCC), line state (LINEX0–Y1, LINEDX, LINEDY, LINESX, LINESY, LINEERR as 16-bit signed, LINEE2, LINEC, LINECNT), rectangle state (RECTX0–Y1, RECTY current row, RECTC).

t64x64.mac - Test Program

Structure

- CP/M program at 0x100H (TPA = Transient Program Area)

- Uses EXTRN to reference all 11 library functions

- Links with the library using L80: L80 T64X64,D64X64,T64X64/N/E

Test Flow

- Print startup banner via BDOS function 9 (print $-terminated string)

- Initialize Dazzler at 0xF000

- Run 7 test suites sequentially: HLIN → VLIN → LINE → CIRC → FCIRC → RECT → FRECT

- Each suite: print header, clear screen to black, run 12 tests, wait for keypress (BDOS function 1)

- After all suites: print "All tests complete!", wait for keypress, disable Dazzler, JMP to WBOOT (warm reboot to CP/M)

Test Design (per suite)

Each suite runs 12 tests structured as 6 cross-quadrant + 6 clipping/edge tests:

- Cross-quadrant: shapes positioned to cross the 32x32 quadrant boundaries (the most error-prone rendering scenario given the memory layout)

- Off-screen/edge: coordinates outside 0–63, passed as unsigned bytes where negative values like −6 are 250 (0xFA). Tests clipping behavior.

Specific notable tests:

- HLIN Test 7: X1=250 (= −6 unsigned), expected to clip to X1=0

- HLIN Test 8: X2=70, expected to clip to X2=63

- VLIN Test 5: Full-height line at X=32 (exactly on the Q0↔Q1 boundary)

- LINE Test 9: Both endpoints off-screen — a horizontal line from (−6, 30) to (80, 30); pixels within [0,63] should appear

- CIRC Test 2: Circle at (32,32) with radius 20 — crosses all four quadrants

- FCIRC Test 6: Large filled circle at center with radius 25 — exercises all four FILLH spans with clipping

Console I/O

Uses only CP/M BDOS:

- Function 1: Console input (wait for key)

- Function 9: Print $-terminated string

Key Design Points

- No floating point, no division hardware — all math is done with shifts, additions, and subtractions. Division by 2 is a right shift; multiplication by 16 is four left shifts.

- 16-bit signed arithmetic on an 8-bit processor — CIRCD (decision parameter) and LINEERR are stored as 2-byte values and manipulated with DAD (16-bit add) and sign extension tricks.

- Register scarcity — 8080 has only A, B, C, D, E, H, L. DZLLINE and DZLCIRC spill all state to memory. DZLHLIN/DZLVLIN use the stack to pass color and the stop coordinate across the pixel-drawing loop.

- The 4-quadrant layout is the defining challenge for this library. Every pixel write must determine which quadrant it falls in and apply the correct memory offset, making even simple primitives non-trivial.

- Negative coordinate convention: callers pass negative values as unsigned bytes ≥ 128. DZLHLIN/DZLVLIN treat ≥ 128 as "clip to 0". DZLPSET treats any coordinate ≥ 64 as out-of-bounds and silently ignores it.

Can you suggest performance optimizations for d64x64.mac, please don't make any changes at this time.

Here are the performance optimizations I'd suggest, ranked roughly by impact:

1. Inline Pixel Writes in DZLHLIN (Highest Impact)

DZLHLIN is the hottest path — it's called by DZLFCIRC (4 times per circle step), DZLFRECT (once per row), and DZLRECT. Currently it calls DZLPSET for every pixel. Each call costs:

- CALL: 17 cycles

- RET: 10 cycles

- PUSH B / POP B (save/restore X,Y): 21 cycles

- PUSH PSW / POP PSW (save/restore color): 21 cycles

- Bounds check: ~20 cycles

That's roughly 89 cycles of overhead per pixel before any actual work is done. For a 64-pixel line, ~5700 cycles of pure overhead.

A direct-write approach for horizontal lines is practical because:

- Y is constant → yo, dzy, and row address are computed once

- X is constant within a quadrant → the byte address simply increments by 1 every 2 pixels

- Every pair of pixels is one byte: even X → low nibble, odd X → high nibble

Strategy: compute the starting byte address once, then loop writing full bytes for the middle section, with partial nibble writes only for the first pixel (if X1 is odd) and last pixel (if X2 is even). This would reduce a 64-pixel line from ~5700 cycles overhead to roughly 15–20 cycles per byte (32 bytes = one full line).

2. Eliminate Stack Thrashing in DZLHLIN / DZLVLIN Loops

Even without inlining, the current loop body at HLLOOP is wasteful:

HLLOOP:

CALL    DZLPSET

POP     D           ; get X2

PUSH    D           ; save it back

MOV     A,B

CMP     D

JZ      HLDONE

JNC     HLDONE

INR     B           ; x++

POP     D           ; get X2 again

POP     PSW         ; get color

PUSH    PSW         ; save color

PUSH    D           ; save X2

JMP     HLLOOP

That's 5 POPs and 3 PUSHes per pixel in the loop body alone (~126 cycles of stack overhead per iteration). Since DZLPSET preserves B and C, both the current X and Y survive each call. Only the color (A) and the stop coordinate (D) are lost. Both can simply be stored in memory variables and reloaded each iteration — two LDA instructions (13 cycles each) replace 126 cycles of push/pop. Same fix applies identically to DZLVLIN.

3. DZLCLR Loop Unrolling

The current clear loop processes one byte per iteration with this overhead structure:

CLRLOOP:

MOV     A,D         ; 5 cycles

MOV     M,A         ; 7 cycles

INX     H           ; 5 cycles

DCX     B           ; 5 cycles

MOV     A,B         ;

ORA     C           ; 9 cycles combined

JNZ     CLRLOOP     ; 10 cycles

~41 cycles per byte. Since 2048 / 8 = 256 exactly, you can unroll 8 writes per iteration and use a single-byte counter (MVI B, 0 = 256 iterations × 8 bytes = 2048):

MVI     B,0         ; 256 iterations

CLRLOOP:

MOV     M,D  ; INX H

MOV     M,D  ; INX H   (repeat 8 times)

...

DCR     B

JNZ     CLRLOOP

This reduces loop overhead from ~34 cycles/byte to ~2 cycles/byte. The DCX B / ORA C / JNZ sequence (24 cycles) is replaced by DCR B / JNZ (15 cycles total) amortized across 8 bytes.

4. dzy×16 Calculation in DZLPSET

The current approach uses four ADD A shifts plus an overflow test and branch:

MOV     A,C         ; dzy

ADD     A           ; ×2

ADD     A           ; ×4

ADD     A           ; ×8

ADD     A           ; ×16

MOV     E,A

MVI     D,0

MOV     A,C

ANI     10H         ; check bit 4 (overflow)

JZ      NOOV

INR     D

NOOV:

DAD     D

12 instructions. Since DAD H doubles HL in 10 cycles and handles 16-bit carry naturally, this simplifies to:

MOV     L,C

MVI     H,0

DAD     H           ; ×2

DAD     H           ; ×4

DAD     H           ; ×8

DAD     H           ; ×16  (HL = dzy×16, correct for dzy 0-31)

6 instructions, no branch, no overflow check needed — the 16-bit DAD handles the carry into H automatically. You'd need to XCHG to preserve the running address total in DE first, then DAD D at the end.

5. Eliminate xo/yo Stack Pushes in DZLPSET

The quadrant offsets xo (0 or 512) and yo (0 or 1024) are currently each pushed to the stack, then retrieved at the end with XCHG / POP H / DAD D / POP D / DAD D. That's 2 pushes and 2 pops (42 cycles) just for bookkeeping.

Since xo + yo can only be 0, 512, 1024, or 1536, you can accumulate a single combined offset in DE as you determine each quadrant:

Start DE = 0

If X >= 32: subtract 32 from X, set DE = 512

If Y >= 32: subtract 32 from Y, add 1024 to DE  (MOV A,D / ADI 4 / MOV D,A)

One push/pop (or none, if you compute the address into HL directly and add DE at the end with DAD D) replaces two push/pop pairs.

6. Add an Internal No-Bounds-Check Variant of DZLPSET

DZLHLIN and DZLVLIN already clip coordinates before calling DZLPSET, so the bounds check inside DZLPSET is redundant for those callers. An internal entry point (e.g., PSET_NC) that skips the two CPI 64 / JNC tests would save ~20 cycles per pixel on every DZLHLIN and DZLVLIN call. DZLCIRC and DZLLINE would continue using the full DZLPSET since they don't pre-clip.

7. Precompute Row Stride in DZLVLIN

For vertical lines, each successive pixel is exactly one row (16 bytes) further in memory — except when crossing the Y=32 quadrant boundary, where the address jumps by 1024 − (31×16) = 528 bytes instead of 16. If you precompute the starting byte address and walk by stride, you replace the full DZLPSET address calculation (roughly 60–70 cycles) with a simple DAD D (10 cycles) per pixel, handling the quadrant crossing as a special case mid-loop.

8. Minor: Register Allocation in DZLCIRC / DZLLINE Inner Loops

Both functions reload the same memory variables repeatedly within a single iteration. For example, in PLOT8, CIRCXC and CIRCYC are each loaded 4 times per call, and CIRCX / CIRCY are each loaded 4 times. Since the 8080 has no registers to spare while also calling DZLPSET, this is hard to eliminate entirely — but precomputing xc+x, xc-x, yc+y, yc-y once at the start of PLOT8 (storing results in memory) and reusing them for all 8 points would eliminate about 16 redundant loads.

Summary Table

| Optimization | Affected Functions | Estimated Gain |
| --- | --- | --- |
| Inline byte writes in DZLHLIN | DZLHLIN, DZLFCIRC, DZLFRECT | 5–10× for fills |
| Eliminate stack thrashing in loops | DZLHLIN, DZLVLIN | ~3× loop speedup |
| DZLCLR unroll | DZLCLR | ~5× |
| dzy×16 via DAD H | DZLPSET | ~6 instructions saved/pixel |
| Combined xo+yo offset | DZLPSET | ~21 cycles/pixel |
| No-bounds-check internal entry | DZLHLIN, DZLVLIN | ~20 cycles/pixel |
| Precompute stride in DZLVLIN | DZLVLIN | ~50 cycles/pixel |
| Reduce redundant LDAs in PLOT8 | DZLCIRC | ~16 LDAs/call |

The first two are by far the most impactful since DZLHLIN sits at the base of the filled circle and filled rectangle render paths, which dominate typical screen-drawing workloads.